Optimize 1D model parameters to match 2D model shapes and frequencies

Introduction
The 1D model contains defined physical properties and the work static analysis solution.

Question

How can you evaluate the correlation between the 1D and 2D models and adjust the physical properties in the 1D model to align its deformed static shapes with the 2D model using the HEEDS-based model update shapes solution process?

Procedure

- In the start folder, open w1d_s.sim and reset the dialog memory.

- Choose Correlation→Correlation group→Analysis Reference to define reference results of the 2D model and importing the ref_2d.op2 reference results.

- Choose Correlation→Model Update group→Shapes to create a model update shapes solution process.

- In the Model Update Shapes dialog box, in the Reference Solution (test or Analysis) group, select Imported Solution.

- Click OK.
In the Simulation Navigator, notice that three shapes are read from result files, and three shape pairs are correlated with the default tolerance.

- In the Simulation Navigator, click the Shape Pairs node and in the Correlation Details View, inspect the list of shape pairs.

- In the Correlation Details View, right-click the first shape pair, and select Correlate Shapes.

- From the Method list, select Error Matrix.

- From the DOF sub-group, select X to specify the direction of the translational DOF to plot.

- In the Error sub-group, select the Absolute Value check box to display the absolute relative error between the reference and work values as a percentage on the error plot.

- Click Close.

- Repeat the steps 8 to 11 and display the absolute relative error for the second pair in the Y direction, and for the third pair in the Z direction.
You can display the three plots in separate viewports.

- Right-click the matrix and select Probing Mode → Single XY.

- From each error metric, create a response for the reference node with ID 3000 by right-clicking it and selecting Create Response.

- In the Simulation Navigator, click the Responses node and in the Correlation Details View, observe the type, DOF, and initial values for each of three responses.

- In the Correlation Details View, select all responses, right-click the selection and choose Add to Objectives.

- In the Simulation Navigator, right-click the Design Variables node, select Create Design Variables and create the following design variables for the BOX beam section:

  - For Area, set Minimum to 5, Maximum to 15 and Increment to 0.2.

  - For Iyy, set Minimum to 220, Maximum to 350, and Increment to 1.

  - For Izz, set Minimum to 8, Maximum to 20, and Increment to 0.2.

- Choose Correlation→Model Update group→Optmize Drop-Down→Options.

- In the Optimization Parameters group, set the Number of Evaluations to 100.

- Click OK.

- In the Simulation Navigator, right-click the Model Update node and choose Optimize.

- Click Start to launch the optimization process to find the best design for your model with specified design variables and objectives.

- Sort designs by the highest performance rating and select a current design with the highest performance rating.

- Observe the changes between the initial and current response values.

- Display the shapes for the first initial shape pair and observe the maximum displacement.

- Display the shapes for the first current shape pair and observe the maximum displacement.

- Repeat steps 25 and 26 for the second and third shape pairs.

You have completed this tutorial.